T-CLOCK Inspection

| Item | What to check | What to look for | Check-off |  |
| --- | --- | --- | --- | --- |
| T - Tires and Wheels |  |  | Front | Rear |
| Tires | Condition | Tread depth, wear, weathering, evenly seated, bulges, imbedded objects | F | R |
|  | Air pressure | Check when cold, adjust to load/speed | F | R |
| Wheels | Spokes | Bent, broken, missing, tension, check at top of wheel: "ring"=ok--"thud"= loose spoke | F | R |
|  | Cast | Cracks, dents | F | R |
|  | Rims | Out of round/true - 5mm, spin wheel, index against stationary pointer | F | R |
|  | Bearings | Grab top and bottom of tire and flex: no free play (click) between hub and axle, no growl when spinning | F | R |
|  | Seals | Cracked, cut, or torn, excessive grease on outside, reddish-brown around outside | F | R |
| C - Controls |  |  |  |  |
| Levers | Condition | Broken, bent, cracked, mounts tight, ball ends on handlebar lever | F | R |
|  | Pivots | Lubricated |  |  |
| Cables | Condition | Fraying, kinks, lubrication: ends and length |  |  |
|  | Routing | No interference or pulling at steering head, suspension, no sharp angles, wire looms on place. |  |  |
| Hoses | Condition | Cuts, cracks, leaks, bulges, chafing, deterioration |  |  |
|  | Routing | No interference or pulling at steering head, suspension, no sharp angles, wire looms on place. |  |  |
| Throttle | Operation | Moves freely, snaps closed, no revving |  |  |
| L - Lights and Electrical |  |  |  |  |
| Battery | Condition | Terminals, clean and tight, electrolyte level, held down securely |  |  |
|  | Vent tube | Not kinked, routed properly, not plugged |  |  |
| Lenses | Condition | Cracked, broken, securely mounted, excessive condensation |  |  |
| Reflectors | Condition | Cracked, broken, securely mounted |  |  |
| Wiring | Condition | Fraying, chafing, insulation |  |  |
|  | Routing | Pinched, interference/pulling at steering head or suspension, wire looms and ties in place |  |  |
| Headlamp | Condition | Cracks, reflector, mounting and adjustment system |  |  |
|  | Aim | Height and right/left |  |  |
| Item | What to check | What to look for | Check-off |  |
| O - Oil and Fluids |  |  |  |  |
| Levels | Engine oil | Check warm as per your Owner’s manual |  |  |
|  | Hypoid gear oil | Transmission, rear driver, shaft |  |  |
|  | Hydraulic Fluid | Brakes, clutch, reservoir or sight glass |  |  |
|  | Coolant | Reservoir and/or coolant recovery tank - cool only |  |  |
|  | Fuel | Tank or gauge |  |  |
| Leaks | Engine oil | Gaskets, housings, seals |  |  |
|  | Hypoid gear oil | Gaskets, seals and breathers |  |  |
|  | Hydraulic Fluid | Hoses, master cylinders, callipers |  |  |
|  | Coolant | Radiator, hoses, tanks, fittings, pipes |  |  |
|  | Fuel | Lines, fuel caps, carbs |  |  |
| C - Chassis |  |  |  |  |
| Frame | Condition | Cracks at gussets, accessory mounts, look for paint lifting |  |  |
|  | Steering-head bearing | No detent or tight spots through full travel, raise front wheel, check for play by pulling/pushing forks |  |  |
|  | Swing arm bushings / bearings | Raise rear wheel, check for play by pulling/pushing swing arm |  |  |
| Suspension | Forks | Smooth travel, equal air pressure/damping anti-drive settings |  |  |
|  | Shock(s) | Smooth travel, equal pre-load/air pressure/damping settings, linkage moves freely and is lubricated |  |  |
| Chain or belt | Tension | Check at tightest point |  |  |
|  | Lubrication | Side plates when hot-DO NOT lubricate belts |  |  |
|  | Sprockets | Teeth not hooked, securely mounted |  |  |
| Fasteners | Threaded | Tight, missing bolts, nuts |  |  |
|  | Clips | Broken, missing |  |  |
|  | Cotter pins | Broken, missing |  |  |
| K - Kickstand |  |  |  |  |
| Side-stand | Condition | Cracks, bent, (safety cut-out switch or pad if equipped) |  |  |
|  | Retention | Springs in place, tension to hold position |  |  |